Paper Rocket Launcher

Tools needed:

- Drill

- Drill Bits ¼ and ½

- Tap and Die Set ¼-18 NPT and bit 7/16” (only need if adding pressure gauge)

- Saw or pipe cutters

- Screw driver

Materials:

- 1 ½” x 5’ PVC

- ¾” x 5’ PVC

- ½” x 2’ PVC

- 1 ½” Plug

- 1 ½” Plug (Thread)

- 1 ½” Tee (Slip x Thread)

- 1 ½” x 1” Tee (Slip x Thread)

- 1” x 3” Nipple

- 3 – 1 ½” Cap

- 4 – ¾” Elbow

- 1” x ½” Bushing (Slip)

- Orbit 1” Inline Sprinkler Valve

- 1” Slip adapters (May come with sprinkler valve)

- Wire Harness with Connector 9V

- Momentary Switch (Normally Off)

- 6’ Landscape Wire 16/2

- 160 PSI ¼ Pressure Gauge (Optional)

- 8 Zip ties

- Wire Nuts

- Tire Valve .453”

- Plumber’s Tape (Yellow for gas)

- Electrical Tape

- PVC Cement

Build Diagram and Instructions:

Launcher:

- Cement 1 ½” cap to 1 ½” x 6” PVC

- Cement 1 ½” x 6” PVC to 1 ½” x 1” Tee (Slip x Thread)

- Cement 1 ½” x 1” Tee (Slip x Thread) to 1 ½” x 6” PVC

- Cement 1 ½” x 6” PVC to 1 ½” Tee (Slip x Thread)

- Drill ½” hole into 1 ½” Plug and pull tire valve through hole

- Cement 1 ½” Plug with tire valve into 1 ½” Tee (Slip x Thread)

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

Drill and thread a ¼” hole into 1 ½” Plug (Thread) using Tap and Die set

- Tape threaded end of pressure gauge and screw into 1 ½” Plug (Thread)

- Tape threaded end of 1 ½” Plug (thread) with pressure gauge and screw into 1 ½” Tee

- Tape threaded ends of 1” nipple and screw into sprinkler valve and 1 ½” x 1” Tee

- Cement 1” x ½” bushing to 1” slip adapter

- Cement ½” PVC to 1” x ½” bushing

- Tape threaded end of 1” slip adapter and screw into sprinkler valve

Launcher Control:

- Connect Sprinkler valve wire to landscaping wire and tape using electrical tape

- Drill holes into each 1 ½” Caps one with a ¼” and the other with a ½”

- Pull wire through the 1 ½” Cap with ¼” hole and 1 ½” x 4” PVC

- Connect landscape wire, momentary switch, and wire harness with connector in series using wire nuts and electrical tape

- Attach switch to 1 ½” Cap with ½” hole

- Dry fit control parts

Launcher Base:

- Cut ¾” x 5’ PVC into 2 - 10” pieces and 2 - 20” pieces

- Dry fit ¾” elbows to ¾” PVC to make the rectangular base

- Mark and drill ¼” holes into ¾” PVC where the launcher will connect to the base

- Pull through zip ties to attach launcher to base

Paper rocket and launch:

- Use the template on the next page to make your paper rocket

- Load your rocket onto the ½” PVC firing end

- Increase the pressure in your chamber by using a bicycle pump or compressor (Note: Do not exceed 50 PSI)

- Stand back and press the launch button.